WHAMO Turbine Integration

Developer Documentation

Software Design & .INP Generation Guide

# 1. Turbine — System Level Concept

A turbine in WHAMO is a hydraulic and mechanical element that converts head (pressure energy) into shaft power. It sits between a headwater reservoir and a tailwater reservoir, extracting energy from the flow.

## 1.1 Network Placement

The correct network topology for a turbine installation is:

Reservoir (HW)  →  Conduit  →  Turbine  →  Tailwater (Reservoir)

Key Rules

- The turbine element must connect exactly two nodes.

- The downstream node MUST be a tailwater reservoir — not a FLOWBC element.

# 2. Turbine in the Software — UI Design

The following elements are already present in the application:

- Reservoir

- Node

- Conduit

- Pump

- FlowBC

A new Turbine element must be added with the following characteristics:

| Property | Specification |
| --- | --- |
| Icon | Similar to pump icon but with reverse energy arrow |
| Directionality | Flow direction matters — must be enforced |
| Element type | Edge element (same category as conduit) |

# 3. Turbine Properties Panel

The properties panel must expose three groups of parameters to the user.

## 3.1 Basic Parameters

| Field | WHAMO Keyword | Description | SI Unit | FPS Unit |
| --- | --- | --- | --- | --- |
| Turbine ID | ID | Unique element name | — | — |
| Type | TYPE | Links to a TCHAR definition | — | — |
| Diameter | DIAMETER | Runner diameter | m | ft |
| Sync Speed | SYNCSPD | Generator synchronous speed | RPM | RPM |
| WR² | WR2 | Moment of inertia | kg·m² | lb·ft² |
| Friction Loss | FRICTION | Mechanical friction loss | kW | HP |
| Windage Loss | WINDAGE | Air resistance / windage loss | kW | HP |

## 3.2 Operation Parameters

| Field | WHAMO Keyword | Description |
| --- | --- | --- |
| Mode | OPTURB | Set to GENERATE for power generation mode |
| Gate Schedule | VSCHEDULE | Gate opening position vs. time curve |

## 3.3 Turbine Characteristics (TCHAR)

Turbine characteristics are defined through a type-based curve system (TCHAR). The TYPE value is shared across turbines, so TCHAR definitions must be maintained in a global store. The following parameters define each characteristic entry:

| Parameter | Description |
| --- | --- |
| Gate (%) | Gate opening percentage — typically 20%, 40%, 60%, 80%, 100% |
| Head | Operating head in ft (FPS) or m (SI) |
| Discharge (Q) | Flow rate corresponding to each gate-head combination |
| Efficiency | Turbine efficiency, expressed as a value between 0 and 1 |

# 4. SI ↔ FPS Unit Conversion

The application must handle unit conversion correctly for all turbine parameters. The conversion factors are defined below.

| Quantity | SI Unit | FPS Unit | Conversion Factor |
| --- | --- | --- | --- |
| Flow | m³/s | CFS (ft³/s) | 1 m³/s = 35.3147 CFS |
| Head | m | ft | 1 m = 3.28084 ft |
| Power | kW | HP | 1 kW = 1.341 HP |
| Diameter | m | ft | 1 m = 3.28084 ft |
| Moment of inertia (WR²) | kg·m² | lb·ft² | Multiply kg·m² × 23.73 |

# 5. Generated .INP File Syntax

The following blocks must be generated as part of a valid WHAMO .INP file. Each block is described below with its required syntax.

## 5.1 TURBINE Block

Defines the physical machine parameters.

TURBINE

ID T1

TYPE 1

SYNCSPD 300

WR2 500000

DIAMETER 5

FRICTION 50.

WINDAGE 10.

FINISH

## 5.2 TCHAR Block

Defines the turbine performance characteristics by gate and head.

TCHAR TYPE 1

GATE 20. 40. 60. 80. 100.

HEAD 200. 150. 100. 50.

Q

<discharge values for each gate-head pair>

EFFICIENCY

<efficiency values for each gate-head pair>

FINISH

## 5.3 OPTURB Block

Defines the operational mode for the turbine.

OPTURB

ID T1

GENERATE

VSCHEDULE 1

FINISH

## 5.4 SCHEDULE Block

Defines the gate opening schedule over time.

SCHEDULE

VSCHEDULE 1

T 0.0  G 100.0

T 10.0 G 0.0

FINISH

# 6. Integration into the Application

The following five steps outline how to add turbine support to the existing application architecture.

## Step 1 — Add Turbine Element

- Add a Turbine tool to the element palette.

- User clicks two nodes to create the turbine edge, following the same interaction pattern as a conduit.

## Step 2 — Open Properties Panel

- When the turbine element is selected, show the properties panel.

- The panel must expose: Basic Parameters, Operation section, and Characteristics section.

## Step 3 — Handle TCHAR Globally

The TYPE field is shared across multiple turbines. TCHAR definitions must be stored in a global application store, not per-element. Example store structure:

{

"tchar": {

"type1": { "gate": [...], "head": [...], "Q": [...], "efficiency": [...] },

"type2": { ... }

}

}

## Step 4 — Validate Before Export

The following conditions must be validated before generating the .INP file:

| Validation Check | Expected Condition |
| --- | --- |
| Turbine connectivity | Must be connected to exactly two nodes |
| Downstream node type | Must be a reservoir (tailwater), not a FLOWBC |
| TYPE reference | Referenced TYPE must exist in the global TCHAR store |
| VSCHEDULE reference | Referenced VSCHEDULE must exist in the SCHEDULE store |

## Step 5 — Generate .INP File

Block order in the generated .INP file is significant. Blocks must appear in the following sequence:

| Order | Block |
| --- | --- |
| 1 | SYSTEM |
| 2 | CONDUITS |
| 3 | RESERVOIRS |
| 4 | TURBINES |
| 5 | TCHAR |
| 6 | OPTURB |
| 7 | SCHEDULE |
| 8 | OUTPUT |
| 9 | CONTROL |

# 7. Common Errors and Causes

The following runtime errors must be surfaced to the user in the UI with clear explanations and corrective guidance.

| WHAMO Error | Root Cause | Corrective Action |
| --- | --- | --- |
| OPERATION NOT SPECIFIED | OPTURB block is missing | Ensure every turbine has an OPTURB block referencing its ID |
| CHARACTERISTICS TYPE NOT SPECIFIED | TCHAR block is missing or TYPE mismatch | Verify the TYPE value matches a defined TCHAR entry |
| FLOW NOT CORRECT | Downstream node is FLOWBC instead of reservoir | Replace the downstream FLOWBC with a tailwater reservoir |
| NO TRANSIENT | Gate schedule is flat (no change over time) | Define a VSCHEDULE with at least two different gate values |

# 8. Supported Use Cases

## Case 1 — Steady State Operation

Gate opening is constant throughout the simulation.

T 0.0  G 100.0

T 999. G 100.0

## Case 2 — Load Rejection

Gate closes from full open to fully closed, simulating a trip event.

T 0.0  G 100.0

T 10.0 G 0.0

## Case 3 — Load Increase

Gate opens from partial to full to simulate increased generation demand.

T 0.0  G 50.0

T 10.0 G 100.0

## Case 4 — Multi-Unit System

Multiple turbine units share the same TCHAR definition (same hydraulic characteristics) but each operates on a different VSCHEDULE, allowing independent gate control per unit.

# 9. Architecture Summary

A complete turbine definition in WHAMO comprises four interdependent blocks. Each block serves a distinct role:

| Block | Role | Scope |
| --- | --- | --- |
| TURBINE | Physical machine definition (dimensions, inertia, losses) | Per element |
| TCHAR | Hydraulic performance curves (gate, head, Q, efficiency) | Global / shared by TYPE |
| OPTURB | Operational mode and schedule reference | Per element |
| SCHEDULE | Time-based gate opening control | Per VSCHEDULE ID |

| Element | Energy Role |
| --- | --- |
| Pump | Adds energy to the system |
| Turbine | Removes energy from the system |
| FlowBC | Forces a prescribed flow boundary condition |
| Reservoir | Sets a fixed head boundary condition |